Audio EQ Cookbook HP15c CE software pac

Pepin Torres, P.E. pepin[dot]torres[at]gmail[dot]com

Introduction:

This software pac calculates all the filters coefficients and related parameters as defined in the classic document Cookbook formulae for audio equalizer biquad filter coefficients.

Who is this for?

This software pac is for any audio enthusiants, audio engineers, or software engineers that need to generate the coefficients for any of the 9 different filters in the cookbook (when there is no computer in sight).  Furthermore, this pac also calculates the complex frequency response of any of the 9 filters at any arbitrary frequency fx. One use-case would be to generate the coefficients of a filter with center frequency f0 and calculate the complex response for a set of frequencies fx (think of being able to plot the response by sampling at different fx frequencies).  Another use-case would be to run this program for a set of filters with different center frequencies f0 and evaluate each filter at a fixed fx to understand the gain/phase contributions of each filter at fx (think of an EQ with multiple bands that overlap fx).

Main Programs:

Program C – Calculate all filter coefficients

Registers required before running: R9, R.0, R.1, R.2

where:

R9 = Fs (sampling frequency in Hz)

R.0 = f0 (center frequency in Hz)

R.1 = Q (set manually, or run Prog A or Prog B to do so automatically,  Q>0)

R.2 = A (see Prog A, A>0)

where f0 is the center frequency of the filter (or cutoff frequency for shelf filters)

Registers modified: R.3, R.4, R.5, R.6, R.7, R.8, R.9

This program will use the parameters in R9, R.0, R.1, and R.2 to calculate all the coeffcients for the 9 digital filters defined in the Cookbook. Given the repetition of the an coeffcients for the Low-Pass, High-Pass, Band-Pass Q peak, Band-Pass 0dB peak, Notch, and All-Pass filters, the a0, a1, and a2 calculation is performed and stored only once.

The output is placed into two matrices, the A matrix containing all the an coeffcients of size [4-by-3] and the B matrix containing all the bn coeffcients of size [9-by-3].

Example:

R9 =  48KHz = 48,000.0

R.0 = f0 =  1KHz = 1,000.0

R.1 = Q = 2.8627260504

R.2 = A  = 1.0

| Key Strokes | Display | Description |
| --- | --- | --- |
| GSB C | 0.000001330 | Stack has no useful data. All results are in Matrix A and Matrix B |

Matrices after running Prog C:

Register states after running:

| R9 | Fs |
| --- | --- |
| R.0 | f0 |
| R.1 | Q |
| R.2 | A |
| R.3 | sin(ω0) |
| R.4 | cos(ω0) |
| R.5 | α |
| R.6 | A-1 |
| R.7 | A+1 |
| R.8 | 2√Aα |
| R.9 | BW |

Program D –  Calculate filter response H(ωx) at arbitrary frequecy fx

Given Matrices A and B as calculated for center frequency f0 at sampling rate Fs, program D will calculate the frequency response of the filter at frequency fx returned as a complex number on the x-register.

Registers needed: All the registers and matrices from running Program C.

Registers modified: R6, R7, R8

For this you need to look up the row index in matrix B corresponding to the filter you want to use.  The following table provides this:

| Filter | Coeffiecients Location Row Matrix A | Coeffiecients Location Row Matrix B |
| --- | --- | --- |
| Low Pass | 1 | 1 |
| High Pass | 1 | 2 |
| Band Pass Q peak gain | 1 | 3 |
| Band Pass 0dB peak gain | 1 | 4 |
| Notch Filter | 1 | 5 |
| All-Pass Filter | 1 | 6 |
| Peaking | 2 | 7 |
| Low Shelf | 3 | 8 |
| High Shelf | 4 | 9 |

Example:

Get frequency response for the Bandpass Filter 0dB peak gain (with center frequency f0 = 1KHz and Fs=48KHz) evaluated at fx = 840Hz:

| t-regsiter | - |
| --- | --- |
| z-register | - |
| y-register | 4 |
| x-register | 840 |

| Key Strokes | Display | Description |
| --- | --- | --- |
| 4 | 4 | Row 4 of Matrix B |
| ENTER | 4.0000 |  |
| 840 | 840 | fx, Frequency of interest |
| GSB D | 0.4971 | Real of H(ωx) |
| f (i) | 0.4999 | Imag of H(ωx) |

Note: Calculator will go into Complex Mode after this program

Running program D results in 3 more matrices being created C, D and E:

Matrix C [3x2] contains the real and imaginary parts of z=ej2πfx/Fs , z-1, and z-2.

Matrix D [4x2] contains the real and imaginary parts of Σ anz-n where n={0,1,2}

Matrix E [9x2] contains the real and imaginary parts of Σ bnz-n where n={0,1,2}

|  | Powers of z for frequency fx = 840.0000 |
| --- | --- |
|  | z^0 = (1.00000000,0.00000000) |
|  | z^-1 = (0.99396096,-0.10973431) |
|  | z^-2 = (0.97591676,-0.21814324) |
|  | Intermediate complex vectors. |
|  | E = sum(bn * zx^-n) D = sum(an * zx^-n) |
| LPF | E(1) 0.016956 + -0.001872j D(1) 0.005551 + 0.004421j |
| HPF | E(2) -0.011954 + 0.001320j D(2) 0.005551 + 0.004421j |
| BPF Q | E(3) 0.001572 + 0.014237j D(3) 0.005551 + 0.004421j |
| BPF 0dB | E(4) 0.000549 + 0.004973j D(4) 0.005551 + 0.004421j |
| NOTCH | E(5) 0.005002 + -0.000552j D(5) 0.005551 + 0.004421j |
| ALL-PASS | E(6) 0.004453 + -0.005525j D(6) 0.005551 + 0.004421j |
| PEAKING | E(7) 0.005551 + 0.004421j D(7) 0.005551 + 0.004421j |
| LOW SHELF | E(8) 0.011102 + 0.008842j D(8) 0.011102 + 0.008842j |
| HIGH SHELF | E(9) 0.011102 + 0.008842j D(9) 0.011102 + 0.008842j |
|  | Frequency response evaluated at fx = 840.0000 [ H(x) = E(wx)/D(wx) ] |
| LPF | H(2*pi*840Hz) = 1.70469312 + -1.69492177j |
| HPF | H(2*pi*840Hz) = -1.20181890 + 1.19493003j |
| BPF Q | H(2*pi*840Hz) = 1.42313489 + 1.43133938j |
| BPF 0dB | H(2*pi*840Hz) = 0.49712577 + 0.49999174j |
| NOTCH | H(2*pi*840Hz) = 0.50287423 + -0.49999174j |
| ALL-PASS | H(2*pi*840Hz) = 0.00574846 + -0.99998348j |
| PEAKING | H(2*pi*840Hz) = 1.00000000 + 0.00000000j |
| LOW SHELF | H(2*pi*840Hz) = 1.00000000 + 0.00000000j |
| HIGH SHELF | H(2*pi*840Hz) = 1.00000000 + 0.00000000j |

The complex-valued H(ωx) in the x-register comes from:

H(fx)  = 	(Σ bn·z-n) / (Σ an·z-n)

=	(E[idxB,1]+jE[idxB,2])/(D[idxA,1]+jD[idxA,2])

Plot of BPF filter with center frequency f0 and transfer function evaluaed at fx

Auxilliary Programs:

Program A – Calculate A

Registers modified: R.1, R.2

This program is used to calculate the A = 10dBgain/40 parameter when designing Peaking, Low-Shelf and High-Shelf filters. Put the gain in dB in x-register and press GSB A.  The A value will be placed in the x-register as well as stored in register R.2 automatically.  If not interested in those filters, put 0.0 in the x-register then press GSB A to populate R.2 (or manually store 1.0 in R.2).

Note:  R.2 > 0.0  is required by the main Program C.

Example: dBGain = 3dB

| Key Strokes | Display | Description |
| --- | --- | --- |
| 3 | 3 | Gain in dB |
| GSB A | 1.1885 | A is also stored in R.2 |

After running:

Q is in R.1

A is in R.2

Program B – Calculate Bandwidth given Q

Registers required before running: R9, R.0

where:

R9 = Fs (sampling frequency in Hz)

R.0 = ω0 = 2πf0/Fs (angular frequency in rad)

This program is used to calculate the filter bandwidth (in multiples of an octave) given Q, f0 , Fs and Q. It also calculate ω0 which is used extensively in Program C.

Example:

Filter center frequency: f0 = 1KHz

Sampling frequency: Fs = 48KHz

Quality Factor: Q = 2.862726050

| Key Strokes | Display | Description |
| --- | --- | --- |
| 0.5 | 0.5 | Bandwidth (in multiples of an octave) |
| GSB B | 1.0 |  |

After running:

Q is stored in R.1

A is stored in R.2

Bandwidth is stored in R.9 (for reference only)

Extra Mini Programs:

These are two very short programs in the .bin file (but not in the listing file) that convert filter gain to and from dB.

Program 0 – Calculate Linear → dB

This program will take the absolute value of the x-register and do 20log10(abs(x-reg))

Program 1 – Calculate dB → Linear

This program will take the absolute value of the x-register and do 10^(x-reg / 20)

Files included:

Below is a listing of the files included in the package and their SHA256.

3f72a6f1ca8649270bee4ca1fefaca92292abbe936cc5a15aa2f24ba661345e6  audio_eq_cookbook.15c

80ab83e487e402f3493d0ffbc7932921eef339068c76729596e633604b703808  audio_eq_cookbook.bin

ff656720283d6aa524ba73588fea86c5bc0eff13eb8c9a59ab67c68f73fd0c89  audio_eq_cookbook.hex

6f43a8fa4ce6195ec19f0e1a038b6923af48e6e2a4a1b486b59e650a1ec8fdab  audio_eq_cookbook.jrn

a077ab03a33a90b114ec45bdb854539d3618bc54d1b2d4c8a036a4868525733d  audio_eq_cookbook.k15

14e75d3ebf8f52c9c462437a46f22628583d05ca5703865ec060324edf45551b  audio_eq_cookbook.m

78b0436cc708674e6de97f937de374dd4c0b9d2cb857df1004330f34b349209a  audio_eq_cookbook.tch

7efa6c698847e7225b5ab6df8746f026a363401810e85d85a00532bc6a37ea61  audio_eq_cookbook.txt

b7e5eccbc65389498ef2039b943de5577fd98f5ca6ef1bf304317c7b3020d3cb  audio_eq_cookbook.xlsx

eab27f0dd13937870cbf154a496d7e7d745584c2ec4c5993426e6e5dfa39bc70  audio_eq_cookbook_quick_start_guide.doc

3f94aa8b49546d295073008bce2a588a616716614e7169545634951a585afea2  audio_eq_cookbook_quick_start_guide.pdf

3d0735608ab95e1b097e591b4334c75d6f5d72a016d6f9b33ab4ebab39f0a9ab  audio_eq_cookbook_users_guide.pdf

Technical Notes:

Given how much program memory this code needs (528 bytes) and the size of the matrices A thru E (568 bytes), the provided image can only be run in 15.2 mode and the author strongly recommends keeping the allocation to the default amount of 19 memory registers.  The programs will not run in default 15 mode and allocating more than 19 registers might cause unexpected behavior up to and including freezing (requiring a hard reset.)

Supporting Docs:

audio_eq_cookbook.xlsx – This sheet has the full program listing and the state of the stack for every instruction.  It also provides a worked out example for a given set of parameters so users can verify all calculations are correct.

audio_eq_cookbook.m – This Matlab/Octave script calculates all the parameters and is meant to be a way to validate the calculator output.  It will plot the filter response and place markers on f0 and fx.

function [C,Hx,Hsweep] = sanity_check(Fs,f0,BW,A,fx)